RTL Circuits: NOR GATE

Overview:

This invertor circuit takes two inputs of either a “HIGH” or “LOW” signal. Dependant on the combination of inputs, the combined signal will be inverted and either illuminate or turn off the LED. The logic circuit is constructed using a combination of transistors and resistors.

Equipment:

- 2× 10kΩ and 1× 1kΩ resistor

- 2× 2N3904 BJTs

- LED

- 5V Linear Breadboard Supply (details in README)

Schematic:

Figure 1: NAND Gate circuit connected to linear power supply

Truth Table:

| Input (X) | Input (Y) | Output (Z) |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 0 |

Circuit Operation

The floating wire inputs connected to the bases of the 2N3904 BJTs can be connected alternately between the +5V rail (logic “HIGH”, “1”) and GND (logic “ZERO”, “0”).

Theoretically, when our combination of input logic is:

- Q1 – Logic 0, Q2 – Logic 0

Both BJTs are switched off and in cut-off operation as the voltage across the base-emitter junction is essentially 0. The BJTs act as open switches, hence the LED is ON.

- Q1 – Logic 0, Q2 – Logic 1

The top transistor (Q1) stays in cut-off and acts as an open switch. The bottom transistor (Q2) behaves as a closed switch as the 5V input provides sufficient base current. This input voltage overcomes the 0.7V base-emitter junction barrier, driving the transistor into saturation. This forms a much lower resistance pathway to GND than the LED, so the main supply current is diverted through Q2, hence the LED is OFF.

- Q1 – Logic 1, Q2 – Logic 0

The bottom transistor (Q2) stays in cut-off and acts as an open switch. The top transistor (Q1) behaves as a closed switch as the 5V input provides sufficient base current. This input voltage overcomes the 0.7V base-emitter junction barrier, driving the transistor into saturation. This forms a much lower resistance pathway to GND than the LED, so the main supply current is diverted through Q2, hence the LED is OFF.

- Q1 – Logic 1, Q2 – Logic 1

Both BJTs receive sufficient base current and are driven into saturation. Together, they form a continuous, low-resistance path to GND. This branch effectively acts as a short circuit, starving the parallel LED branch of current and pulling the output node LOW, so the LED is OFF.

Experimental Calculations

| Scenario | VCE / V |
| --- | --- |
| 1 | 2.5 |
| 2 | 0.0088 |
| 3 | 0.0109 |
| 4 | 0.0083 |

In states where output logic is LOW, measured voltages are significantly lower than standard saturation voltages across the collector-emitter junction (0.2V) [1].

For deep saturation, the maximum load current is required. We can use the equation IC=VCC-VCERC. Hence, we can approximate the collector current to be 4.99mA.

The base current can be given as IB=VCC-VBERB. As our standard base-emitter voltage is given as 0.7V, we can calculate the base current as 0.43mA.

Our forced beta (βforced) can be defined as the collector current divided by our base current, which gives 11.6, which is significantly lower than a standard DC Current Gain for a 2N3904 BJT at 150 [1]. This shows that there is too much base current compared to the collector current, forcing it into deep saturation, so the transistor acts as almost ideal switch.

References

[1]	‘2N3903 - General Purpose Transistors’. Accessed: Jul. 11, 2026. [Online]. Available: https://www.onsemi.com/download/data-sheet/pdf/2n3903-d.pdf