RTL Circuits: NOT GATE

Overview:

This invertor circuit takes an input of either a “HIGH” or “LOW” signal and flips it. The logic circuit is constructed using a combination of transistors and resistors.

Equipment:

- 10kΩ and 1kΩ resistor

- 2N3904 BJT

- LED

- 5V Linear Breadboard Supply (details in README)

Schematic:

Figure 1: NOT Gate circuit schematic connected to linear power supply

Truth Table:

| Input (X) | Output (Y) |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

Circuit Operation:

The floating wire input connected to the base of  the 2N3904 BJT can be connected alternately between the +5V rail (logic “HIGH”, “1”) and GND (logic “ZERO”, “0”).

As we have the emitter as our shared reference point, this circuit can be simplified into a common emitter switch.

Theoretically, when our input wire is connected to:

- 5V Rail (Logic 1)

IB=VCC-VBERB
“VBE”, which is the voltage drop across the base-emitter junction, for a 2N3904 BJT, it is usually set at 0.7V. “VCC”, which is our 5V from the power rail, and “RB”, which is our input resistance to the base of the transistor, 10kΩ. Hence our expected “IB” is 430μA.

ICsat=VCC-VCEsatRC

To calculate the saturation current, we can calculate the collector current when the transistor acts as a short circuit, assuming it to be ideal, hence “VCEsat” is 0V. In this case, “VCC”, which is our 5V from the power rail, and “RC”, which is collector pull-up resistor of the transistor, 1kΩ. Hence “ICsat”  is 5mA.

To verify this BJT’s operation, we can calculate the Forced Beta (βforced) by dividing the collector current by the base current, which is approximately 11.63. This is massively smaller than the natural minimum beta (hFE) of 2N3904 transistors, and so it was forced into saturation. The LED is off.

Figure 2: NOT gate with Logic 1 input

- GND (Logic 0)

When our input wire is connected to GND, there is no input voltage at the base, hence “IB” is 0A. To allow current to flow, the voltage drop across the base-emitter junction must be 0.7V. As this is not the case, the threshold is not overcome, hence the transistor is off, operating in cut-off.

As a result, no current can flow through the collector of the transistor. Hence, current must flow through the collector pull-up resistor and directly to the load, in this case, an LED.

An unloaded output should sit at 5V. Due to the presence of the LED, a portion of the 5V is taken for the LED forward voltage, which varies depending on the colour of the LED.

Figure 3: Not gate with Logic 0 input

Experimental Measurements

- Base-Emitter Activation: The base-emitter voltage drop required to forward-bias the transistor was measured at 0.717V closely aligning with the theoretical 0.7V.

- Logic 0 Output (Saturation): With a 5V input, the transistor successfully entered hard saturation. However, a physical BJT is never a mathematically perfect short circuit. The measured output voltage was 44mV, proving the gate successfully choked the load current while sinking the remainder to ground.

- Logic 1 Output (Diode Clamping): With a 0V input, the transistor successfully entered cut-off. Instead of the theoretical unloaded 5V, the output clamped at 2.64V. This is due to the physical load; the wide-bandgap semiconductor material of the blue LED dictates a high forward voltage drop to generate photons, pulling the collector voltage down. This reading changed to about 1.82V with a red LED.